Konfigurasi site BSD

- Mikrotik

/ip ipsec profile

set [ find default=yes ] dpd-interval=5s nat-traversal=no

add dh-group=modp1024 dpd-interval=5s enc-algorithm=des name=profile1 \

nat-traversal=no

/ip ipsec peer

add address=10.10.55.28/32 name=peer-tunnel-to-forti profile=profile1

/ip ipsec proposal

add enc-algorithms=des name=proposal1 pfs-group=none

/ip ipsec identity

add peer=peer-tunnel-to-forti

/ip ipsec policy

add dst-address=172.16.10.0/24 peer=peer-tunnel-to-forti proposal=proposal1 \

src-address=172.16.20.0/24 tunnel=yes

/ip route

add disabled=no dst-address=172.16.20.0/24 gateway=1.1.1.2 routing-table=main \

suppress-hw-offload=no

add disabled=no dst-address=0.0.0.0/0 gateway=10.10.55.254 routing-table=main \

suppress-hw-offload=no

/ip firewall nat

add action=masquerade chain=srcnat disabled=yes out-interface=ether1

add action=masquerade chain=srcnat ipsec-policy=out,none

==========================================================================

/ip address

add address=1.1.1.1/30 comment=TO-LAN interface=ether2 network=1.1.1.0

add address=10.10.55.121/24 interface=ether1 network=10.10.55.0

- Cisco Switch

ip dhcp pool Local-LAN

network 172.16.20.0 255.255.255.0

default-router 172.16.20.1

dns-server 8.8.8.8 8.8.4.4

interface Ethernet0/0

no switchport

ip address 1.1.1.2 255.255.255.252

!

interface Ethernet0/1

description TO-LAN

no switchport

ip address 172.16.20.1 255.255.255.0

ip route 0.0.0.0 0.0.0.0 1.1.1.1

Konfigurasi site BEKASI

- Fortigate

Konfigurasi IP Local

Konfigurasi WAN/Internet

Konfigurasi IPSEC

Konfigurasi Firewall Policy

Local to Internet, agar local dapat akses internet

Konfigurasi local to VPN, berfungsi saat ipsec sudah jadi masing2 local dapat terhubung

Konfigurasi Policy  Mikrotik to VPN, berfungsi untuk site Bekasi dial IpSec dapat langsung diterima di Fortigate

Testing jika establish makan local dari site BSD to Bekasi dapat terhubung, jika belum kalian bisa cek encrypt dari kedua Device dan juga pastikan IP Publiknya dapat terhubung. Thanks